Sherwood Engineering HF Test Results

Model
Yaesu FT-2000
Serial # 6L080075 

Test Date: 7/19/2007 
 


IF BW 6000 –6 / -60, kHz



Ultimate


dB

IF BW 2400 –6 / -60, kHz
2.32 / 3.35

Ultimate
80 noise
dB

IF BW 1800 –6 / -60, kHz



Ultimate


dB

IF BW 500 –6 / -60, Hz  
500 / 840

Ultimate
80 noise
dB

Front End Selectivity (A – F)










IF Rejection, 14.2 MHz @
 kHz IF





dB

First IF Rejection @  
 MHz IF






dB

Dynamic Range 100 kHz


dB


IP3

dBm

Dynamic Range 20 kHz 6k roofing
90
dB


IP3

dBm

Dynamic Range 20 kHz 3k roofing
81
dB


IP3

dBm

Dynamic Range 5 kHz


dB


IP3

dBm

Dynamic Range 2 kHz 6k roofing
61
dB


IP3

dBm

Dynamic Range 2 kHz 3k roofing
63
dB


IP3

dBm

Blocking above noise floor at 100 kHz spacing, AGC On

120

dB

Phase noise (normalized) at 10 kHz spacing:



122

dBc

Noise floor, SSB bandwidth 14 MHz, Preamp Off


-117

dBm

Noise floor, SSB bandwidth 14 MHz, Preamp 1 On


-124

dBm

Noise floor, SSB bandwidth 14 MHz, Preamp 2 On


-134

dBm

Sensitivity at 14 MHz, Preamp Off)




1.0

uV

Sensitivity at 14 MHz, Preamp 1 On




0.3

uV

Sensitivity at 14 MHz, Preamp 2 On




0.14

uV
Noise floor, 500 Hz, 14.2 MHz, Preamp Off



-122

dBm

Noise floor, 500 Hz, 14.2 MHz, Preamp 1 On


-132

dBm

Noise floor, 500 Hz, 14.2 MHz, Preamp 2 On


-140

dBm
Noise floor, SSB, 2 MHz







dBm

Noise floor, CW, 2 MHz







dBm

Sensitivity, 2 MHz








uV

Signal for S9, Preamp Off / On / On




135 / 37 / 14
uV

Preamp(s), dB gain, 






11 / 20

dB

AGC threshold at 3 dB, Preamp Off





5
uV

AGC threshold at 3 dB, Preamp 1 On




 
1.3
uV
AGC threshold at 3 dB, Preamp 2 On





0.5
uV
Rob Comments:
Just a note:

Blocking numbers are at 100 kHz, and the weak signal is below the AGC threshold.

None of the S5 non-sense that has the AGC messing with the blocking numbers.

The AGC is on, but not functional due to level.

That is a change I made a few years ago when Elecraft started publishing blocking with the AGC turned off.

